Comprehensive Study: Solar PV + Battery Storage System for a Juice Shop in Egypt

PART 1: ENERGY CONSUMPTION ANALYSIS

1.1 Appliance Load Profile

| Appliance | Qty | Power (W) | Hours/Day | Daily Energy (kWh) |
| --- | --- | --- | --- | --- |
| Refrigerators (small) | 3 | 439 | 24 | 31.61 |
| Refrigerators (large) | 2 | 645 | 24 | 30.96 |
| Ice Cream Freezers | 3 | 330 | 24 | 23.76 |
| Roller Machine | 1 | 2,200 | 6 | 13.20 |
| LED Lights | 25 | 12 | 15 | 4.50 |
| Blender | 1 | 1,500 | 2 | 3.00 |
| Machine | 1 | 750 | 2 | 1.50 |
| Ice Crusher | 1 | 500 | 2 | 1.00 |
| TOTAL |  |  |  | 109.53 kWh/day |

1.2 Key Metrics

| Metric Valu | Value |
| --- | --- |
| Daily Consumption | 109.53 kWh |
| Monthly Consumption | 3,286 kWh |
| August Bill (Reference) | EGP 8,279 |
| August Consumption (Reference) | 2,421 kWh |
| Effective Electricity Rate | EGP 3.42/kWh |
| Current Estimated Monthly Bill | EGP 11,238 |

1.3 Egypt-Specific Critical Factors (2025–2026)

Net Metering Cancellation:

- Egypt suspended new net metering applications as of March 2025

- Complete halt of new applications effective December 31, 2025

- Impact: Cannot sell excess power to the grid; battery storage becomes essential

Peak-Hour Surcharge:

- 20% surcharge on commercial consumption exceeding 250 kWh/month during peak hours

- Peak rate: EGP 2.64/kWh vs. standard EGP 2.20/kWh

- Impact: Battery can shave peak consumption and save the differential

Commercial Tariff (April 2026):

| Tier | Old Price (EGP/kWh) | New Price (EGP/kWh) | Increase |
| --- | --- | --- | --- |
| 0–100 kWh | 0.85 | 1.62 | 91% |
| 101–250 kWh | 1.68 | 2.16 | 29% |
| 251–600 kWh | 2.20 | 2.64 | 20% |
| 601–1000 kWh | 2.60 | 2.74 | 5% |
| 1000 kWh | 2.79 | 2.79 | 0% |

Government Support:

- Egypt granted Golden Licenses to battery storage projects worth $800 million

- Banque Misr offers green financing up to 100% with repayment up to 10 years

PART 2: FUNDAMENTAL EQUATIONS

2.1 Solar System Sizing

System Size (kW) = Daily Consumption ÷ (Peak Sun Hours × Performance Ratio)

- Peak Sun Hours (Cairo): 5.4 hours/day

- Performance Ratio: 0.80 (accounts for heat, dust, wiring losses)

2.2 Daily Energy Production

Daily Production (kWh) = System Size × Peak Sun Hours × Performance Ratio

2.3 Battery Sizing

Required Capacity = Critical Load × Backup Hours ÷ Depth of Discharge

- Depth of Discharge (LiFePO4): 80%

- Critical Load: ~4.0 kW (refrigerators, freezers, lights)

2.4 Annual Savings

Annual Savings = Monthly Production × 12 × Electricity Rate

2.5 Payback Period

Payback Period (years) = Total System Cost ÷ Annual Savings

2.6 Load Shifting Savings

Load Shifting Savings = Daily Discharge × 365 × (Usage Rate − Charging Rate ÷ Round-Trip Efficiency)

- Round-Trip Efficiency: 90%

2.7 Net Present Value (NPV)
The Net Present Value (NPV) is a financial metric used to evaluate the profitability of an investment by comparing the present value of all future cash inflows to the initial investment.

It accounts for the time value of money-the idea that a dollar today is worth more than a dollar in the future because it can be invested and earn a return.

NPV = t=1nNet Cash Flowt(1+r)t-Intial Investment

- R: discount rate 10% (required rate of return, opportunity cost of capital)

- T: Year number (1,2,3,.....)

- N: Number of years (in our case: 10)

- Initial investment = Total system cost at year 0

- 1(1+r)t : this term is the discount factor. It converts future cash flows into their present value.

- Net Cash Flowt: Cash flow (saving) minus cash outflow (maintenance costs) in year t.

PART 3: ALL SCENARIOS — DETAILED ANALYSIS

SCENARIO 1: Solar Only — 25 kW (No Battery)

Design:

- System Size: 25 kW (46 panels × 550W)

- Daily Production: 108 kWh

- Load Coverage: ~99%

- Battery: None

Costs:

| Component | Estimated Cost (EGP) |
| --- | --- |
| Solar Panels (25 kW) | 180,000 – 212,000 |
| Grid-Tied Inverter | 100,000 – 180,000 |
| Mounting, Cabling, Installation | 60,000 – 95,000 |
| Total | 340,000 – 487,000 |
| Midpoint | ~413,500 |

ROI:

- Monthly Savings: ~EGP 11,000

- Annual Savings: ~EGP 132,000

- Payback Period: 3–4 years

10-Year Cash Flow:

| Year | Savings | Maintenance | Net Cash Flow | PV (10%) |
| --- | --- | --- | --- | --- |
| 0 | — | — | (413,500) | (413,500) |
| 1 | 132,000 | 8,270 | 123,730 | 112,482 |
| 2 | 142,650 | 8,940 | 133,710 | 110,445 |
| 3 | 152,636 | 9,566 | 143,070 | 107,446 |
| 4 | 163,320 | 10,236 | 153,084 | 104,556 |
| 5 | 174,753 | 10,952 | 163,801 | 101,720 |
| 6 | 186,986 | 11,719 | 175,267 | 98,851 |
| 7 | 200,075 | 12,539 | 187,536 | 96,206 |
| 8 | 214,080 | 13,417 | 200,663 | 93,710 |
| 9 | 229,066 | 14,356 | 214,710 | 91,037 |
| 10 | 245,100 | 15,361 | 229,739 | 88,679 |
| NPV |  |  |  | +691,632 |

Assessment: Best financial returns BUT not feasible due to net metering cancellation. Excess power cannot be exported.

SCENARIO 2: Battery Only — 10 kWh (No Solar)

Design:

- Battery: 10 kWh LiFePO4

- Inverter: 3 kW hybrid

- Solar: None

Costs:

| Component | Estimated Cost (EGP) |
| --- | --- |
| Battery 10 kWh | 80,000 – 150,000 |
| Hybrid Inverter 3 kW | 15,000 – 45,000 |
| Installation & Accessories | 20,000 – 35,000 |
| Total | 115,000 – 230,000 |
| Midpoint | ~172,500 |

ROI:

- Electricity Savings: EGP 0

- Backup Duration: ~2.5 hours

- Payback: Never (reliability investment only)

Assessment: Pure reliability investment. No financial return.

SCENARIO 3: Solar 10 m² + Battery 10 kWh

Design:

- Area: 10 m²

- Panels: 3 × 670W (LONGi Hi-MO X10, 24.8% efficiency)

- System Size: 2.01 kW

- Daily Production: 8.7 kWh

- Coverage: ~8%

Costs:

| Component | Estimated Cost (EGP) |
| --- | --- |
| Solar Panels (2.01 kW) | 16,700 – 17,900 |
| Hybrid Inverter 3 kW | 15,000 – 45,000 |
| Battery 10 kWh | 80,000 – 150,000 |
| Mounting, Cabling, Installation | 25,000 – 45,000 |
| Total | 136,700 – 257,900 |
| Midpoint | ~197,300 |

ROI:

- Monthly Savings: ~EGP 893

- Annual Savings: ~EGP 10,716

- Payback: ~18.4 years

10-Year Cash Flow:

| Year | Savings | Maintenance | Net Cash Flow | PV (10%) |
| --- | --- | --- | --- | --- |
| 0 | — | — | (197,300) | (197,300) |
| 1 | 10,716 | 3,946 | 6,770 | 6,154 |
| 2 | 11,581 | 4,266 | 7,315 | 6,045 |
| 3 | 12,392 | 4,565 | 7,827 | 5,879 |
| 4 | 13,259 | 4,884 | 8,375 | 5,721 |
| 5 | 14,187 | 5,226 | 8,961 | 5,565 |
| 6 | 15,180 | 5,592 | 9,588 | 5,408 |
| 7 | 16,243 | 5,983 | 10,260 | 5,264 |
| 8 | 17,380 | 6,402 | 10,978 | 5,127 |
| 9 | 18,597 | 6,850 | 11,747 | 4,981 |
| 10 | 19,899 | 7,330 | 12,569 | 4,851 |
| NPV |  |  |  | −143,300 |

Assessment: Negative NPV. Not economically viable as standalone. Best as Phase 1 for future expansion.

SCENARIO 4: Solar 25 m² + Battery 10 kWh

Design:

- Area: 25 m²

- Panels: 7 × 670W

- System Size: 4.69 kW

- Daily Production: 20.3 kWh

- Coverage: ~19%

Costs:

| Component | Estimated Cost (EGP) |
| --- | --- |
| Solar Panels (4.69 kW) | 32,830 |
| Hybrid Inverter 5 kW | 25,000 – 75,000 |
| Battery 10 kWh | 80,000 – 150,000 |
| Mounting, Cabling, Installation | 35,000 – 65,000 |
| Total | 172,000 – 322,000 |
| Midpoint | ~247,000 |

ROI:

- Monthly Savings: ~EGP 2,083

- Annual Savings: ~EGP 24,996

- Payback: ~10 years

10-Year Cash Flow:

| Year | Savings | Maintenance | Net Cash Flow | PV (10%) |
| --- | --- | --- | --- | --- |
| 0 | — | — | (247,000) | (247,000) |
| 1 | 24,996 | 4,940 | 20,056 | 18,233 |
| 2 | 27,013 | 5,340 | 21,673 | 17,911 |
| 3 | 28,904 | 5,714 | 23,190 | 17,419 |
| 4 | 30,927 | 6,114 | 24,813 | 16,950 |
| 5 | 33,092 | 6,542 | 26,550 | 16,488 |
| 6 | 35,408 | 7,000 | 28,408 | 16,022 |
| 7 | 37,887 | 7,490 | 30,397 | 15,594 |
| 8 | 40,539 | 8,014 | 32,525 | 15,190 |
| 9 | 43,377 | 8,575 | 34,802 | 14,757 |
| 10 | 46,414 | 9,175 | 37,239 | 14,374 |
| NPV |  |  |  | −94,062 |

Assessment: Marginal. Negative NPV but approaching breakeven. Reasonable for moderate budgets.

SCENARIO 5: Solar 50 m² + Battery 20 kWh ( RECOMMENDED)

Design:

- Area: 50 m²

- Panels: 15 × 670W

- System Size: 10.05 kW

- Daily Production: 43.4 kWh

- Coverage: ~40%

- Battery Backup: 4 hours for critical loads

Costs:

| Component | Estimated Cost (EGP) |
| --- | --- |
| Solar Panels (10.05 kW) | 70,350 |
| Hybrid Inverter 10 kW | 50,000 – 150,000 |
| Battery 20 kWh | 160,000 – 300,000 |
| Mounting, Cabling, Installation | 50,000 – 85,000 |
| Total | 330,000 – 605,000 |
| Midpoint | ~467,500 |

ROI:

- Monthly Savings: ~EGP 4,453

- Annual Savings: ~EGP 53,436

- Payback: ~8.4 years

10-Year Cash Flow:

| Year | Savings | Maintenance | Net Cash Flow | PV (10%) |
| --- | --- | --- | --- | --- |
| 0 | — | — | (467,500) | (467,500) |
| 1 | 53,436 | 9,350 | 44,086 | 40,078 |
| 2 | 57,749 | 10,108 | 47,641 | 39,373 |
| 3 | 61,791 | 10,815 | 50,976 | 38,284 |
| 4 | 66,116 | 11,572 | 54,544 | 37,260 |
| 5 | 70,744 | 12,382 | 58,362 | 36,243 |
| 6 | 75,696 | 13,249 | 62,447 | 35,227 |
| 7 | 80,995 | 14,176 | 66,819 | 34,283 |
| 8 | 86,665 | 15,168 | 71,497 | 33,389 |
| 9 | 92,732 | 16,230 | 76,502 | 32,434 |
| 10 | 99,223 | 17,366 | 81,857 | 31,597 |
| NPV |  |  |  | −108,832 |

Assessment: Best practical balance of cost, savings, and reliability. Shortest payback among battery-inclusive scenarios.

SCENARIO 6: Solar 10 m² + Battery 30 kWh

Design:

- Area: 10 m²

- Panels: 3 × 670W (2.01 kW)

- Battery: 30 kWh LiFePO4

- Inverter: 5 kW hybrid

Costs:

| Component | Estimated Cost (EGP) |
| --- | --- |
| Solar Panels (2.01 kW) | 26,700 – 37,900 |
| Hybrid Inverter 5 kW | 25,000 – 75,000 |
| Battery 30 kWh | 240,000 – 450,000 |
| Installation & Accessories | 20,000 – 35,000 |
| Total | 311,700 – 597,900 |
| Midpoint | ~454,800 |

ROI with Peak Shaving:

- Solar Self-Consumption: ~EGP 8,860/year

- Peak Shaving: ~EGP 1,285/year

- Load Shifting: ~EGP 4,730/year

- Total Annual Savings: ~EGP 14,875

- Payback: ~30.6 years

10-Year Cash Flow:

| Year | Savings | Maintenance | Net Cash Flow | PV (10%) |
| --- | --- | --- | --- | --- |
| 0 | — | — | (454,800) | (454,800) |
| 1 | 14,875 | 9,096 | 5,779 | 5,254 |
| 2 | 16,075 | 9,833 | 6,242 | 5,158 |
| 3 | 17,200 | 10,521 | 6,679 | 5,016 |
| 4 | 18,404 | 11,257 | 7,147 | 4,881 |
| 5 | 19,692 | 12,045 | 7,647 | 4,749 |
| 6 | 21,070 | 12,888 | 8,182 | 4,615 |
| 7 | 22,545 | 13,790 | 8,755 | 4,491 |
| 8 | 24,123 | 14,755 | 9,368 | 4,374 |
| 9 | 25,812 | 15,788 | 10,024 | 4,251 |
| 10 | 27,619 | 16,893 | 10,726 | 4,138 |
| NPV |  |  |  | −407,873 |

Assessment: Poor financial returns. Oversized battery relative to solar generation.

SCENARIO 7: Solar 50 m² + Battery 50 kWh

Design:

- Area: 50 m²

- Panels: 15 × 670W (10.05 kW)

- Battery: 50 kWh LiFePO4

- Inverter: 10 kW hybrid

- Backup: ~10 hours critical loads

Costs:

| Component | Estimated Cost (EGP) |
| --- | --- |
| Solar Panels (10.05 kW) | 70,350 |
| Hybrid Inverter 10 kW | 50,000 – 150,000 |
| Battery 50 kWh | 400,000 – 750,000 |
| Mounting, Cabling, Installation | 50,000 – 85,000 |
| Total | 570,350 – 1,055,350 |
| Midpoint | ~812,850 |

ROI:

- Solar Self-Consumption: ~EGP 44,200/year

- Load Shifting: ~EGP 11,900/year

- Peak Shaving: ~EGP 1,600/year

- Total Annual Savings: ~EGP 57,700

- Payback: ~14.1 years

10-Year Cash Flow:

| Year | Savings | Maintenance | Net Cash Flow | PV (10%) |
| --- | --- | --- | --- | --- |
| 0 | — | — | (812,850) | (812,850) |
| 1 | 57,700 | 16,257 | 41,443 | 37,675 |
| 2 | 62,358 | 17,576 | 44,782 | 37,013 |
| 3 | 66,722 | 18,811 | 47,911 | 35,984 |
| 4 | 71,392 | 20,128 | 51,264 | 35,010 |
| 5 | 76,389 | 21,537 | 54,852 | 34,061 |
| 6 | 81,736 | 23,044 | 58,692 | 33,107 |
| 7 | 87,457 | 24,657 | 62,800 | 32,217 |
| 8 | 93,579 | 26,383 | 67,196 | 31,380 |
| 9 | 100,129 | 28,230 | 71,899 | 30,484 |
| 10 | 107,138 | 30,206 | 76,932 | 29,690 |
| NPV |  |  |  | −524,119 |

Assessment: High investment with weak financial return. Value is in energy independence and extended backup.

SCENARIO 8: Solar 10 m² + Battery 50 kWh

Design:

- Area: 10 m²

- Panels: 3 × 670W (2.01 kW)

- Battery: 50 kWh LiFePO4 (5 × 10 kWh units)

- Inverter: 5 kW hybrid

- Backup: ~10 hours critical loads

Costs:

| Component | Estimated Cost (EGP) |
| --- | --- |
| Solar Panels (2.01 kW) | 17,000 – 18,000 |
| Hybrid Inverter 5 kW | 25,000 – 75,000 |
| Battery 50 kWh | 400,000 – 750,000 |
| Installation & Accessories | 15,000 – 25,000 |
| Total | 457,000 – 868,000 |
| Midpoint | ~662,500 |

ROI:

- Annual Savings: ~EGP 14,900 (solar + load shifting)

- Payback: ~44.5 years

10-Year Cash Flow:

| Year | Savings | Maintenance | Net Cash Flow | PV (10%) |
| --- | --- | --- | --- | --- |
| 0 | — | — | (662,500) | (662,500) |
| 1 | 14,900 | 13,250 | 1,650 | 1,500 |
| 2 | 16,102 | 14,326 | 1,776 | 1,468 |
| 3 | 17,230 | 15,326 | 1,904 | 1,430 |
| 4 | 18,436 | 16,399 | 2,037 | 1,391 |
| 5 | 19,727 | 17,546 | 2,181 | 1,354 |
| 6 | 21,108 | 18,774 | 2,334 | 1,316 |
| 7 | 22,585 | 20,088 | 2,497 | 1,281 |
| 8 | 24,166 | 21,494 | 2,672 | 1,248 |
| 9 | 25,858 | 22,999 | 2,859 | 1,212 |
| 10 | 27,668 | 24,609 | 3,059 | 1,180 |
| NPV |  |  |  | −649,520 |

Assessment: Worst financial scenario. Unbalanced system — battery too large for solar array.

PART 4: COMPREHENSIVE SCENARIO COMPARISON

| # | Scenario | Cost (EGP) | Annual Savings (EGP) | Payback | Coverage | Backup | NPV (10 yr) | Rating |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Solar 25 kW only | 413,500 | 132,000 | 3.1 yr | 99% | None | +691,632 | (not feasible) |
| 2 | Battery 10 kWh only | 172,500 | 0 | Never | 0% | 2.5 hr | −210,000 |  |
| 3 | Solar 10 m² + Batt 10 kWh | 197,300 | 10,716 | 18.4 yr | 8% | 2.5 hr | −143,300 |  |
| 4 | Solar 25 m² + Batt 10 kWh | 247,000 | 24,996 | 10 yr | 19% | 4 hr | −94,062 |  |
| 5 | Solar 50 m² + Batt 20 kWh | 467,500 | 53,436 | 8.4 yr | 40% | 4 hr | −108,832 |  |
| 6 | Solar 10 m² + Batt 30 kWh | 454,800 | 14,875 | 30.6 yr | 8% | 6–8 hr | −407,873 |  |
| 7 | Solar 50 m² + Batt 50 kWh | 812,850 | 57,700 | 14.1 yr | 40% | 10 hr | −524,119 |  |
| 8 | Solar 10 m² + Batt 50 kWh | 662,500 | 14,900 | 44.5 yr | 8% | 10 hr | −649,520 |  |

PART 5: IMPLEMENTATION PLANS

5.1 Recommended Scenario (Scenario 5): 50 m² + 20 kWh Battery

Phase 1: Assessment & Preparation (2 weeks)

1. Conduct comprehensive energy audit — measure actual loads for one week

2. Confirm exact available roof area (50 m²) and check for shading

3. Obtain 3 quotations from certified suppliers

4. Separate critical loads (refrigerators, freezers, lights) from non-critical loads in electrical panel

Phase 2: Financing (2–4 weeks)

- Option A: Banque Misr green loan (up to 100%, repayment up to 10 years)

- Option B: Self-funding

- Option C: Mixed (30–40% down payment + financing)

Phase 3: Installation (3–4 weeks)

1. Install mounting structure and solar panels (15 × 670W)

2. Install 10 kW hybrid inverter

3. Install 20 kWh LiFePO4 battery (2 × 10 kWh units)

4. Install Automatic Transfer Switch (ATS) for critical load separation

5. Install smart monitoring system for remote performance tracking

Phase 4: Operation & Maintenance

| Task | Frequency | Annual Cost (EGP) |
| --- | --- | --- |
| Solar panel cleaning | Bi-weekly | 6,000 – 12,000 |
| Battery inspection | Every 6 months | 2,000 – 4,000 |
| Inverter & cabling inspection | Annually | 3,000 – 5,000 |
| Total |  | 11,000 – 21,000 |

Option 1: 2 Phased Expansion Plan (Starting with 10 m²)

| Phase | Area | System | Estimated Cost (EGP) | Coverage |
| --- | --- | --- | --- | --- |
| 1 | 10 m² | 2 kW solar + 10 kWh battery | ~197,000 | 8% |
| 2 | 25 m² | Add 4 panels + 10 kWh battery | ~130,000 | 19% |
| 3 | 50 m² | Add 8 panels + 10 kWh battery | ~180,000 | 40% |

Option 2: Installation Plan for Scenario 7 (50 m² + 50 kWh)

Phase 1: Assessment & load prioritization (2 weeks)

Phase 2: Financing (2–4 weeks)

Phase 3: Installation (3–5 weeks)

- Install 15 solar panels on 50 m²

- Install 10 kW hybrid inverter

- Install 50 kWh battery (5 × 10 kWh units or integrated units)

- Install ATS for critical load separation

- Install bidirectional inverter and DC breaker

- Install smart monitoring system

PART 6: COMMERCIAL VALUE OF LARGE BATTERIES (30–50 kWh)

6.1 Beyond Direct ROI

1. Inventory Protection (Most Important Commercial Value)

- Critical load: ~4 kW (refrigerators + freezers)

- 50 kWh battery = ~10 hours backup = full working day coverage

- Potential inventory loss during 10-hour outage: thousands of EGP (ice cream, fresh juice, fruits)

2. Energy Independence

- 50 kWh battery + 10 kW solar = near-independent system during day and evening

- Can run all equipment (roller machine, blender, ice crusher) during peak hours without grid draw

3. Grid Outage Resilience

- Large battery allows continued operation during extended outages

- No need to reduce loads or shut down equipment

4. Green Financing Eligibility

- Banque Misr offers up to 100% financing with up to 10-year repayment

- Larger battery makes project more bankable

6.2 When to Choose a Large Battery (30–50 kWh)

Choose a large battery only if:

- High-value inventory: Potential loss from 10-hour outage exceeds EGP 50,000–100,000

- Future expansion planned: 50 kWh battery can accommodate future solar expansion (up to 20 kW)

- Concessional financing obtained: Monthly installment (~EGP 5,000–7,000) < monthly savings (~EGP 4,800)

- Peak tariff spread widens: If differential exceeds EGP 0.80–1.00/kWh, large battery becomes more viable

PART 7: FINAL RECOMMENDATIONS

7.1 Primary Recommendation

Scenario 5: Solar 50 m² + Battery 20 kWh is the optimal choice:

- Cost: ~EGP 467,500 (midpoint)

- Annual Savings: ~EGP 53,436

- Payback: ~8.4 years

- Coverage: 40% of consumption

- Backup: 4 hours for critical loads

7.2 If Roof Area is Limited to 10 m²

- Scenario 3 (10 m² + 10 kWh) as Phase 1, then expand incrementally

- Do not invest in 30–50 kWh battery with only 10 m² solar — unbalanced and uneconomical

7.3 If Maximum Independence is Required

- Scenario 7 (50 m² + 50 kWh) provides 10-hour backup but weak financial return

- Accept that large battery is an insurance premium, not a profitable investment

7.4 Key Strategic Considerations

- Net metering cancellation makes battery storage essential — you must store your own solar energy for nighttime use

- Peak-hour surcharge (20%) creates value for battery peak-shaving

- Banque Misr financing makes large systems accessible with manageable monthly payments

- Regular maintenance is critical — dust in Egypt reduces output by 5–15%

- Avoid diesel generators — operating cost (~EGP 1.48/kWh) is uneconomical vs. solar + battery

PART 8: SUMMARY TABLE — ALL SCENARIOS

| Metric | Value |
| --- | --- |
| Current Consumption | 3,286 kWh/month |
| Current Bill | ~EGP 11,238/month |
| Effective Rate | EGP 3.42/kWh |
| Best Financial Scenario | Solar 25 kW only (not feasible) |
| Best Practical Scenario | Solar 50 m² + Battery 20 kWh |
| Best for Independence | Solar 50 m² + Battery 50 kWh |
| Worst Financial Scenario | Solar 10 m² + Battery 50 kWh |
| Recommended Payback Target | 8–10 years |
| System Lifespan | 20–25 years (panels), 10–15 years (batteries) |

PART 9: CONCLUSION

A solar-plus-battery system is a sound investment for your juice shop despite the limited roof area. The combination of high Egyptian electricity tariffs, excellent solar resources (5.4 PSH), and the need for reliable power creates a compelling case.

Key points:

- Scenario 5 (50 m² + 20 kWh) offers the best balance of cost, savings, and reliability with an 8.4-year payback

- Net metering cancellation makes batteries essential — you cannot rely on grid export

- Peak-hour surcharge adds value to battery storage through peak shaving

- Large batteries (30–50 kWh) are reliability assets, not short-term financial investments

- Phased implementation allows you to start small and expand as budget and roof area permit

- Green financing from Banque Misr makes the investment accessible

After the payback period, you will enjoy nearly free electricity for the remaining 15–20 years of the system's life, with only minor maintenance costs.